LAB READ ME & CONFIGURATION GUIDE

Cisco Packet Tracer — Automated DHCP Service Implementation

1. Lab Overview & Objectives

This documentation guides you through configuring a standard automated Dynamic Host Configuration Protocol (DHCP) server infrastructure on a Cisco ISR4321 router. The topology consists of an edge router providing direct IP pooling services through a Layer 2 Multilayer Switch to an end-user workstation client (PC0).

| Lab Parameters / Requirement Spec | Value / Target Assignment |
| --- | --- |
| 1) DHCP Excluded Address Range | 10.1.1.1 to 10.1.1.100 |
| 2) DHCP Pool Identity Name | pc (Case-Sensitive) |
| 3) Subnet Network Scope | 10.1.1.0 /24 (255.255.255.0) |
| 4) Default Gateway Mapping | Router1 Lan Interface IP (e.g., 10.1.1.254) |
| 5) Domain Name Server (DNS) | Router1 Address Reference |
| 6) Validation Checkpoint | PC0 must lease IP and ping Router1 Loopback |

2. Cisco IOS Device Configuration Steps

Step 2.1: Local Interface IP Setup

Before launching the DHCP system pool daemon, the native router physical interface connected to the switch fabric must possess an active, static IPv4 address within the corresponding subnet network mask zone:

Router1> enable Router1# configure terminal Router1(config)# interface GigabitEthernet0/0/0 Router1(config-if)# ip address 10.1.1.254 255.255.255.0 Router1(config-if)# no shutdown Router1(config-if)# exit

Step 2.2: Loopback Interface Creation

To satisfy objective requirement #6 (testing loopback structural reachability), instantiate a local logical virtual loopback interface on Router1:

Router1(config)# interface Loopback0 Router1(config-if)# ip address 1.1.1.1 255.255.255.255 Router1(config-if)# exit

Step 2.3: DHCP Server Engine Instantiation

Execute the pool allocation syntax commands. Ensure the case-sensitive pool string identifier exactly matches your lab constraints:

Router1(config)# ip dhcp excluded-address 10.1.1.1 10.1.1.100 Router1(config)# ip dhcp pool pc Router1(dhcp-config)# network 10.1.1.0 255.255.255.0 Router1(dhcp-config)# default-router 10.1.1.254 Router1(dhcp-config)# dns-server 10.1.1.254 Router1(dhcp-config)# end Router1# copy running-config startup-config

3. Workstation PC0 IP Configuration

By default, endpoint network interfaces in Packet Tracer initialize with static allocation profiles. To bind to the new server pool:

Open Workspace Node: Click directly on the graphical workstation icon labeled PC0.

Navigate to Network Profile: Select the Desktop tab module header and select the IP Configuration tool applet.

Modify IP Assignment Mode: Toggle the operational selection radio option from Static to DHCP.

ℹ️ AUTOMATED BACKGROUND OPERATIONAL SUMMARY: When switching to DHCP, Packet Tracer broadens a broadcast discovery packet (DHCPDISCOVER). The router answers with a lease offer. Within moments, the interface will automatically populate with the first available non-excluded index: 10.1.1.101, subnet mask 255.255.255.0, and primary default gateway pointer set to 10.1.1.254.

4. Diagnostics & Verification Checks

Resolving Empty 'show ip dhcp binding' Table Tables

If executing the address assignment tracking command yields an empty response list summary, investigate using this diagnostics matrix checklist sequence:

1. Confirm Port Physical Link Status: Verify that link lights spanning across Router1, Switch0, and PC0 display as stable solid green colors. If they are amber or red, verify the physical interfaces are up using show ip interface brief.

2. Inspect Pool Allocation Exclusions: Ensure your excluded scope boundaries do not mistakenly envelop your host network segment. If you configured ip dhcp excluded-address 10.1.1.1 10.1.1.254, no addresses are left for clients.

3. Re-trigger client interface request: Cycle PC0's interface profile toggle back to Static and then reset it to DHCP to force a clean handshake attempt.

Verification CLI Command Reference

To view active client table mappings on Router1, utilize the administrative binding diagnostic utility command:

Router1# show ip dhcp binding IP address Client-ID/Hardware address/User name Lease expiration Type 10.1.1.101 0060.2FA4.DE01 -- Automatic

End-To-End Loopback Connectivity Test

To finalize verification, open PC0's desktop Command Prompt terminal and attempt to ping the virtual loopback address hosted on Router1:

PC> ping 1.1.1.1 Pinging 1.1.1.1 with 32 bytes of data: Reply from 1.1.1.1: bytes=32 time<1ms TTL=255 Reply from 1.1.1.1: bytes=32 time<1ms TTL=255 Reply from 1.1.1.1: bytes=32 time<1ms TTL=255 Reply from 1.1.1.1: bytes=32 time<1ms TTL=255 Ping statistics for 1.1.1.1: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)